DRAFT for review: confirm the cool-down rates, the burner shut-down sequence, the fans to keep running and the lock-out rules against the furnace and burner manufacturers' manuals and the site safety rules, then approve.

| MASS WIRE AND STEELS PVT. LTD. · BHIWADI Work instruction · Furnace shut-down | DOCUMENT WI-HT-03 | REVISION 0 | APPROVED __ / __ / ____ |
| --- | --- | --- | --- |

| Covers | Equipment shut-down (CQI-9 HTSA 1.15) |
| --- | --- |
| Applies to | Furnaces F-01, F-02, F-03 and F-04: hardening chamber, controlled cooling chamber, tempering chamber; quench tanks F-01 (water) and F-02 (oil) |
| Used by | Furnace operator, Loading / unloading operator, Shift supervisor, Maintenance technician, HT Manager |
| Record | Shutdown checksheet F-HT-03A; shut-down record on the CALOR Alarms page |
| Kept at | Control panel of each furnace; HT office; Procedures & WIs page in CALOR |
| Language | English, Hindi |

# 1. Purpose

Stop a furnace line safely with every tray of product accounted for: either run out of the line with its full thermal cycle, or recorded as left inside and held for a decision.

# 2. Scope

Every planned stop of a pusher furnace line (no orders, holiday, maintenance, hot standby) and the end of a stop after an emergency once the line is safe. Emergencies themselves are SOP-HT-11; start-up is WI-HT-02. This instruction does not replace the furnace and burner manufacturers' manuals.

# 3. Responsibilities

- Shift supervisor: plans the stop, decides run-out or product left inside with the HT Manager, signs the checksheet.

- Furnace operator: runs the line out, records the trays and times, shuts the burners down by the sequence, fills in the checksheet.

- Maintenance technician: gas isolation and lock-out for maintenance, checks of the burners, fans and drives.

- HT Manager: agrees product left inside and decides its disposition.

# 4. Safety

- Burners off only by the burner management sequence; gas valves closed; for maintenance the gas and electrical isolation are locked and tagged by the authorised person.

- The chambers, trays, quench oil and doors stay hot for many hours after the stop; the F-02 oil quench is a fire risk until it is cool and covered.

- Gas smell at any time: stop, do not switch anything, follow SOP-HT-11 (EI-HT-05 part B).

- Heat-resistant gloves, face shield, apron and safety shoes; keep clear of moving equipment during the run-out.

# 5. Types of shut-down

| Type of shut-down | What is different |
| --- | --- |
| Planned stop (no orders, holiday) | All steps; the line is run out empty; burners off; gas valves closed. Check of the hearths for fallen parts once cool. |
| Planned maintenance stop | All steps plus lock-out / tag-out of gas, electrical drives and burners for the maintenance work; record the work so the restart (WI-HT-02) can check the TUS / SAT needs on the Equipment page. |
| Hot standby (short idle) | Steps A–C and D (hot standby): the line is run out empty, the burners stay on at the standby set values under the burner management system; no product until a restart by WI-HT-02 with a soak. |
| Emergency stop (power, gas, fire, alarm) | First SOP-HT-11 (the emergency instruction). When the line is safe, finish with steps C–G of this instruction; the trays left inside are SUSPECT. |
| Breakdown (equipment fault, line cannot run out) | Steps A, C–G; record every tray left inside and its position (step 10); the HT Manager decides on it. |

# 6. Steps

Key points: ⚠ Safety · ● Critical (checksheet item: stop if not met) · ◆ Quality

| No. | Step | Key point |
| --- | --- | --- |
| A. Plan the stop (shift supervisor) |  |  |
| 1 | Agree the stop and its type with the Production Manager and the HT Manager (table in section 5); write the reason on the Shutdown checksheet F-HT-03A. | ◆ Quality |
| 2 | Decide whether the line is run out empty (normal) or product stays inside (only when the HT Manager agrees, e.g. a short stop to hot standby). | ● Critical |
| 3 | Identify the last batch to be loaded: part, batch, heat and its last tray no. on the Daily production page. | ● Critical |
| 4 | Inform maintenance of the stop time; for a maintenance stop agree the work, the lock-out points and who holds the permits. | ⚠ Safety |
| B. Run the line out |  |  |
| 5 | Stop loading product after the last tray of the last batch. Load one empty separator tray after it. | ◆ Quality |
| 6 | Keep pushing at the push time of the last batch's job card with EMPTY trays until the last product tray has left the tempering chamber: 13 pushes after it to leave the hardening chamber and 12 more to leave the tempering chamber — 25 empty trays in all, counting the separator tray of step 5. Never change the push time or a set value while product is still inside. | ● Critical |
| 7 | Enter the empty trays on the Daily production page so the last product tray's exit time is calculated; record the actual time the last product tray left the tempering chamber. | ◆ Quality |
| 8 | Quench (F-01 / F-02) and controlled cooling stay within the job card until the last product tray has passed them. | ● Critical |
| 9 | Count the pieces of the last lot at unloading and compare with the pieces loaded; do the lot changeover / trap point check for the last lot (CALOR → Trap points). | ● Critical |
| 10 | Product cannot be run out (stop by an alarm, a jam or a decision): record every tray left inside and its position on the checksheet; it is SUSPECT until the HT Manager decides (SOP-HT-08). | ● Critical |
| C. Quench and controlled cooling |  |  |
| 11 | Controlled cooling chamber empty; switch its blower off after the last tray (isothermal annealing). |  |
| 12 | F-02 oil quench: after the last tray keep the oil circulation and cooler running as the procedure says until the oil is below its safe temperature; then close the hood / cover the tank (fire risk). Never leave hot oil without its cover. | ⚠ Safety |
| 13 | F-01 water quench: circulation as the procedure says; cover the tank. |  |
| 14 | Quench elevator / conveyor empty and in its parking position. | ⚠ Safety |
| D. Burners off |  |  |
| 15 | Hot standby: lower the set values to the standby values of the furnace instruction; keep the burners on under the burner management system; no product is loaded until a restart by WI-HT-02. | ◆ Quality |
| 16 | Full stop: lower the set values at the cool-down rate of the furnace manual if it gives one (refractory); then shut the burners down by the burner management sequence. | ⚠ Safety |
| 17 | Close the gas safety shut-off valves of the furnace; for maintenance close and lock the furnace gas isolation valve (tag with the name and date). | ⚠ Safety |
| 18 | Keep the combustion air blower and the exhaust / flue fan running as the furnace manual requires to cool the burners and clear the furnace; switch them off only when the manual allows. | ⚠ Safety |
| 19 | Keep SCADA recording running at least until the burners are off and the chamber temperatures are falling; note the time the burners went off. | ◆ Quality |
| E. Make the line safe |  |  |
| 20 | Doors closed for slow cooling unless the furnace manual says otherwise. | ◆ Quality |
| 21 | Pusher, extractor and rail transfer in their safe position; hydraulic pump off; selectors to OFF. | ⚠ Safety |
| 22 | For maintenance: electrical isolation and lock-out of the drives, blowers and burners by the authorised person (lock-out / tag-out); work only with a permit. | ⚠ Safety |
| 23 | Write the handover note to maintenance / the next shift: state of the furnace, gas valves, locks, quench, anything abnormal seen during the run-out. |  |
| F. When the furnace is cool enough |  |  |
| 24 | Check the hearth, rails and tray positions of both chambers and the controlled cooling chamber for fallen parts; record the check on the Trap points page (parts found: identify the lot, quarantine). | ● Critical |
| 25 | Inspect the trays and fixtures that ran in the last batch (CALOR → Load build); remove cracked or distorted ones. | ◆ Quality |
| 26 | Use the stop for the planned maintenance items of the predictive maintenance checksheet F-MT-01 (items on WATCH / PLAN WORK). |  |
| G. Records |  |  |
| 27 | Complete the Shutdown checksheet F-HT-03A: result SHUTDOWN COMPLETE, signatures. | ◆ Quality |
| 28 | Record the shut-down on CALOR → Process → Alarms (type, reason, last tray, burners off, product left inside). | ◆ Quality |
| 29 | Record the stop on the Daily production page as a pause of the push cycle (reason) when the line will restart with trays inside, or leave the day closed when the line was run out empty. | ◆ Quality |

# 7. When something goes wrong

A product tray cannot leave the line, a burner or gas valve does not close, or the quench oil overheats: stop the run-out, inform the shift supervisor and maintenance, follow SOP-HT-11, record the trays left inside and start a reaction plan on the CALOR Alarms page.

# 8. Records

- Shutdown checksheet F-HT-03A (kept 3 years or as the record retention rule says).

- Shut-down record on CALOR → Process → Alarms; the last tray and its exit time on the Daily production page; the trap point check on the Trap points page.

- Lock-out / permit records of the maintenance work.

# 9. Related documents

WI-HT-01 General operating procedure · WI-HT-02 Furnace start-up · SOP-HT-11 Emergency response · SOP-HT-08 Segregation of suspect and nonconforming product · Trap point register (CALOR) · F-HT-03A Shutdown checksheet · F-MT-01 Predictive maintenance checksheet · Furnace and burner manufacturers' manuals · Site lock-out / tag-out procedure.

# 10. Revisions

| Rev | Approved | Approved by | Reason for the change |
| --- | --- | --- | --- |
| 0 |  |  | First issue |

| Prepared by (HT Engineer) Sign · Date | Approved by (HT Manager) Sign · Date | Controlled copy issued to Location · Date |
| --- | --- | --- |