DRAFT for review: confirm every step against the furnace and burner manufacturers' manuals, the gas safety rules and the site emergency plan, fill in the contacts, then approve.

| MASS WIRE AND STEELS PVT. LTD. · BHIWADI Procedure · Emergency response for the heat treat line | DOCUMENT SOP-HT-11 | REVISION 0 | APPROVED __ / __ / ____ |
| --- | --- | --- | --- |

| Covers | Methods of addressing potential emergencies, such as power failure (CQI-9 HTSA 1.15) |
| --- | --- |
| Applies to | The heat treat line: furnaces F-01 to F-04 (hardening chamber, controlled cooling chamber, tempering chamber), quench tanks F-01 (water) and F-02 (oil), pusher, extractor, rail transfer, SCADA |
| Used by | Furnace operator, Loading / unloading / quench operator, Shift supervisor, Maintenance technician, HT Engineer, HT Manager |
| Kept at | Control panel of each furnace (with the quick reference card, section 5); quench area; HT office; Procedures & WIs page in CALOR |
| Replaces | The separate drafts EI-HT-04 to EI-HT-07 on the Procedures & WIs page (same numbers kept as sections) |

# 1. Purpose

Make sure that in every foreseeable emergency on the heat treat line the people are safe first, the furnace and the quench are brought to a safe state, and every tray that may not have had its full thermal cycle is identified, held and decided on before it can reach a customer.

# 2. Scope

All emergencies that can interrupt or upset the heat treatment process: power failure, gas supply failure or leak, burner flame failure, fire, quench failure, combustion air blower failure, controlled cooling failure, mechanical jam of the pusher / extractor / transfer, control / SCADA / thermocouple failure and over-temperature trip. Medical emergencies, flooding and site-wide events follow the site emergency plan; this procedure covers the heat treat side of them.

# 3. Responsibilities

- Furnace operator: takes the immediate actions, notes the time and the tray positions, tags suspect trays, records the event.

- Shift supervisor: takes charge at the line, calls maintenance and the emergency team, decides to stop / restart the line with the HT Manager.

- Maintenance technician / Maintenance Head: makes the equipment safe, finds and removes the cause, confirms the equipment is fit to restart.

- HT Manager: decides the disposition of the suspect product, approves the restart after a stopping emergency, keeps this procedure current.

- Quality Manager: releases or rejects held product with the HT Manager; checks that no suspect tray was dispatched.

# 4. General rules for every emergency

- Priorities: 1. people · 2. gas and fire safety · 3. the quench and controlled cooling (most critical product) · 4. the furnace · 5. the record.

- Never bypass or reset a safety interlock, flame safeguard, gas pressure switch or over-temperature trip before the cause is known.

- Never push a tray into a chamber, quench or cooling chamber that is not within the job card.

- Note the time the emergency started and ended and the position of every tray. The Daily production page gives the tray in each position from the last tray entered.

- Every tray that was in the line during an emergency is SUSPECT until the HT Manager decides (red HOLD tag, quarantine area, SOP-HT-08). Rework only by the rework procedure SOP-QA-12.

- Record every emergency on the CALOR Alarms page (alarm type, start, acknowledgement, end, what was done, product affected, reaction plan) and every stop of pushing as a pause of the push cycle on the Daily production page. The second-level data review (HTSA 1.9) checks these records within 24 h.

- Restart after a stopping emergency only by WI-HT-02 with the Start-up checksheet F-HT-02A.

# 5. Quick reference card (post at each control panel)

| Emergency | Instruction | First actions (then follow the instruction) |
| --- | --- | --- |
| Power failure | EI-HT-04 | Burners off, gas valves closed · F-02 tray above oil: lower it / keep clear · selectors OFF · note time and tray positions · tag all trays |
| Gas fault / flame failure | EI-HT-05 A | Valves closed · do not reset · stop pushing · maintenance |
| Gas smell / leak | EI-HT-05 B | No switches, no flames · main gas valve closed from outside · evacuate · emergency team |
| Fire | EI-HT-06 | Fire alarm · stop line · gas off · oil fire: hood + CO₂ / foam, NEVER water · evacuate |
| Quench out of limits | EI-HT-07 | Stop pushing · check agitation / temperature / level · no tray into the quench |
| Air blower / exhaust fan failure | EI-HT-08 | Burners off on interlock · stop pushing · maintenance |
| Controlled cooling failure | EI-HT-09 | Stop pushing · note the tray in the chamber and its cooling minutes |
| Pusher / transfer jam | EI-HT-10 | Emergency stop · never force · doors closed · maintenance |
| Thermocouple / SCADA failure | EI-HT-11 | Compare both walls · manual readings every 15 min · stop pushing if no valid reading |
| Over-temperature trip | EI-HT-12 | Do not reset · find the cause · tag every tray of the chamber |

# 6. Emergency contacts (fill in and keep current)

| Who | Name | Phone (write in) |
| --- | --- | --- |
| Shift supervisor |  |  |
| Maintenance Head |  |  |
| HT Manager |  |  |
| Electrical maintenance / electricity board |  |  |
| Gas supplier emergency |  |  |
| Site emergency team / fire brigade (101) |  |  |
| Ambulance (108) / first aid |  |  |
| Quality Manager |  |  |

# 7. Emergency instructions

Key points: ⚠ Safety · ● Critical (stop if not met) · ◆ Quality

# EI-HT-04  Power failure

| Recognised by | Lights / panels off, motors stopped (pusher, blowers, quench agitator, pumps), burners off, SCADA on UPS or blank. |
| --- | --- |
| Alarm on the CALOR Alarms page | Power failure |
| Furnaces | F-01, F-02, F-03, F-04 |

Immediate actions

| No. | Action | Key point |
| --- | --- | --- |
| First minute: people and gas |  |  |
| 1 | Raise the alarm to the shift supervisor and maintenance; note the time of the failure (watch / phone). | ⚠ Safety |
| 2 | Check that every burner is off and the gas safety shut-off valves have closed (valve position indicators). Never open a gas valve by hand. | ⚠ Safety |
| 3 | Gas smell or a valve that did not close: follow EI-HT-05 part B (gas leak) at once. | ⚠ Safety |
| 4 | Keep everybody away from the quench elevator, the extractor and the doors: they may move without warning when the power returns. Put the selector switches of the pusher, extractor and elevator to OFF / manual. | ⚠ Safety |
| Quench and controlled cooling (the most critical trays) |  |  |
| 5 | F-02 oil quench: a tray hanging above or half in the oil is a fire risk. Lower it fully into the oil with the manual / emergency drive of the elevator if the manual allows it; otherwise keep the area clear and have the CO₂ / foam extinguisher ready. Close the quench hood. | ⚠ Safety |
| 6 | F-02: oil agitation and circulation have stopped – a tray in the oil quenches slowly. Note the tray, the time it entered the oil and the time agitation stopped. | ◆ Quality |
| 7 | F-01 water quench: the water pump / agitation stopped – note the tray in the quench and the time. | ◆ Quality |
| 8 | Controlled cooling chamber (isothermal annealing): the cooling blower stopped – note the tray in the chamber and how many minutes of its cooling time had passed. | ◆ Quality |
| Furnace |  |  |
| 9 | Keep the furnace doors closed to hold the heat; do not open inspection ports. | ◆ Quality |
| 10 | Combustion air / exhaust: follow the furnace manual (some burners need the blower to protect them from back-heat; restart it first when the power returns). | ⚠ Safety |
| 11 | Standby supply (if the site has a generator / UPS – to confirm): start it by its instruction for the critical loads only: control power and SCADA, burner management, quench agitation and oil circulation, exhaust. | ⚠ Safety |
| Record and tag |  |  |
| 12 | Write down the position of every tray at the failure: the 13 in the hardening chamber, the one in the quench or controlled cooling chamber, the 12 in the tempering chamber (Daily production page: the last tray entered gives the positions). | ◆ Quality |
| 13 | Tag all of them SUSPECT (red HOLD tag) when they come out, by tray number (SOP-HT-08). | ● Critical |
| 14 | Record the failure on the Alarms page (alarm “Power failure”, start, end, trays at risk) and the pause of the push cycle on the Daily production page (reason: power failure). | ◆ Quality |
| When the power returns |  |  |
| 15 | Restart by WI-HT-02 (restart after power failure): purge, relight by the burner management sequence, never by hand. | ⚠ Safety |
| 16 | Do not push until every zone of both chambers is back within tolerance on both wall thermocouples and soaked (Start-up checksheet F-HT-02A). | ● Critical |

Product in the line

| Where the tray was | Risk and disposition |
| --- | --- |
| Hardening chamber zone 1 (preheat) | Low risk; tag and include in the release test of the batch. |
| Hardening chamber zones 2–3 (austenitizing) | Below the austenitizing temperature during the failure: incomplete austenitizing possible. Hold; hardness (and microstructure where specified) on samples from these trays before release. Long hot hold after the restart: also check decarburization. |
| F-02 tray in the oil quench / on the elevator | Slack quench (no agitation) or delayed quench: highest risk of soft spots and low core hardness. Hold; 100 % hardness or sort; re-harden by the rework procedure (SOP-QA-12) only with the HT Manager's release. |
| F-01 tray in the water quench | Quench without agitation: soft spots, uneven hardness, distortion / cracks. Hold; hardness and crack check. |
| Controlled cooling chamber (IA) | Cooling interrupted: wrong transformation (hardness / microstructure outside the specification). Hold; hardness and microstructure. |
| Tempering chamber (tempering / isothermal hold / normalizing) | Cooled below the set value then reheated: usually acceptable but not proven. Hold; hardness on samples of each batch before release. |

Restart only when

- Power stable and the cause known (maintenance / electricity board).

- Burners relit by the burner management sequence; all zones within tolerance and soaked; quench and controlled cooling within the job card.

- Start-up checksheet F-HT-02A completed and the HT Manager has decided what happens to the trays at risk.

# EI-HT-05  Gas supply failure, flame failure or gas leak

| Recognised by | Burner lockout lamp, flame failure alarm, gas pressure switch trip, zone temperature falling; or the smell of gas. |
| --- | --- |
| Alarm on the CALOR Alarms page | Gas pressure low / high · Burner flame failure |
| Furnaces | F-01, F-02, F-03, F-04 |

Immediate actions

| No. | Action | Key point |
| --- | --- | --- |
| Part A – gas pressure fault or burner lockout |  |  |
| 1 | Note the time and the burners / zones affected; inform the shift supervisor and maintenance. | ⚠ Safety |
| 2 | Check that the safety shut-off valves of the affected burners closed. Do not reset the lockout or the pressure switch before the cause is known. | ⚠ Safety |
| 3 | Stop pushing when a zone leaves its tolerance (both walls); keep the doors closed. Record the pause of the push cycle. | ● Critical |
| 4 | Maintenance checks the gas supply pressure, the filter and regulator, the igniter and the flame sensor of the burner. |  |
| 5 | Relight only after the purge, by the burner management sequence (WI-HT-02). | ⚠ Safety |
| Part B – gas leak (smell of gas, hissing, gas detector alarm) |  |  |
| 6 | Do not switch anything electrical on or off near the leak; no flames, no phones in the area. | ⚠ Safety |
| 7 | Close the main gas isolation valve from a safe place outside the bay (location: ______________). | ⚠ Safety |
| 8 | Evacuate the bay to the assembly point; call the site emergency team and the gas supplier (numbers in section 6). | ⚠ Safety |
| 9 | Ventilate (open doors from outside); nobody re-enters until the gas supplier / maintenance declares it safe. | ⚠ Safety |
| Both cases |  |  |
| 10 | Tag the trays in the zones that left their tolerance as suspect (SOP-HT-08); record the alarm on the Alarms page. | ◆ Quality |

Product in the line

| Where the tray was | Risk and disposition |
| --- | --- |
| Zones that left their tolerance | Hold the trays that were in those zones from the alarm until the zone was back in tolerance; hardness before release. |
| Gas leak with evacuation | Treat as a power failure of the whole line: every tray in the line is suspect (table of EI-HT-04). |

Restart only when

- Cause found and corrected; gas pressure normal; no leak (leak test by maintenance / the gas supplier after a leak).

- Purge and relight by the burner management sequence; zones within tolerance and soaked.

# EI-HT-06  Fire (quench oil, a door, hydraulic oil)

| Recognised by | Flames or smoke at the F-02 oil quench, a door, the hydraulic unit or a cable tray; fire alarm. |
| --- | --- |
| Alarm on the CALOR Alarms page | Fire / flames at the quench or a door |
| Furnaces | F-01, F-02, F-03, F-04 |

Immediate actions

| No. | Action | Key point |
| --- | --- | --- |
| 1 | Raise the fire alarm; call the site emergency team / fire brigade. | ⚠ Safety |
| 2 | Stop pushing and the quench elevator; close the main gas isolation valve from a safe place. | ⚠ Safety |
| 3 | F-02 quench oil fire: close the quench hood / lid; use the CO₂ / foam system or extinguishers. NEVER water on an oil fire. | ⚠ Safety |
| 4 | Hydraulic oil fire: stop the hydraulic pump (emergency stop), CO₂ / dry powder. | ⚠ Safety |
| 5 | Electrical fire: isolate the panel if it can be done safely; CO₂ only. | ⚠ Safety |
| 6 | Evacuate by the site emergency plan; count everybody at the assembly point. | ⚠ Safety |
| 7 | No restart before the safety review by the Maintenance Head and the HT Manager. | ⚠ Safety |
| 8 | Tag every tray in the line and in the quench as suspect (SOP-HT-08); record the event on the Alarms page. | ◆ Quality |

Product in the line

| Where the tray was | Risk and disposition |
| --- | --- |
| Every position | Treat as a power failure of the whole line (table of EI-HT-04); the tray in the burning quench: scrap unless the HT Manager decides otherwise after tests. |

Restart only when

- Fire brigade / emergency team has released the area.

- Damage inspected; quench oil tested (flash point, water, contamination) before use; electrical and hydraulic checks done.

- Start-up by WI-HT-02 with the Start-up checksheet.

# EI-HT-07  Quench failure (F-01 water, F-02 oil)

| Recognised by | Quench temperature out of the job card limits, agitator / pump stopped or its current falling (agitation is not monitored: check the agitator current and the surface movement), level low, crackle test positive (oil), cooler / cooling water failure. |
| --- | --- |
| Alarm on the CALOR Alarms page | Quench oil temperature out of limits (also used for the F-01 water temperature) · Quench agitation failure · Quench oil level low · Cooling water failure |
| Furnaces | F-01 (water), F-02 (oil) |

Immediate actions

| No. | Action | Key point |
| --- | --- | --- |
| 1 | Stop pushing at once: no tray into a quench out of its limits. | ● Critical |
| 2 | Check agitation (motor running, current, surface movement), temperature, level and the cooler / cooling water; inform maintenance. |  |
| 3 | F-02 oil above its upper limit: fire risk – keep the hood closed, no tray into the oil, cooling to the limit before restart. | ⚠ Safety |
| 4 | F-02 water in the oil (crackle test / Karl Fischer): stop quenching; never add water to the oil; heat-out or replace the oil by the quench media procedure. | ⚠ Safety |
| 5 | F-01 water too hot: restore cooling water / the cooling tower before quenching. | ◆ Quality |
| 6 | Tag the trays quenched since the last good reading as suspect (SOP-HT-08); record the alarm on the Alarms page and the out-of-limit test on the Quench media page. | ◆ Quality |

Product in the line

| Where the tray was | Risk and disposition |
| --- | --- |
| Trays quenched since the last good quench check | Hold; hardness (core and surface where specified) and crack check before release. |
| Trays still in the hardening chamber | Not affected while they stay in the chamber; do not push them out until the quench is within the limits. Long stop: decarburization check. |

Restart only when

- Quench within the job card limits (temperature, level, agitation confirmed by current and surface movement); oil test passed where the failure involved water or overheating.

# EI-HT-08  Combustion air blower or exhaust fan failure

| Recognised by | Air pressure switch trip, burners off on the air interlock, blower or exhaust fan stopped or noisy, current abnormal, flue gases at the doors. |
| --- | --- |
| Alarm on the CALOR Alarms page | Combustion air blower failure / low air pressure · Exhaust / flue fan failure |
| Furnaces | F-01, F-02, F-03, F-04 |

Immediate actions

| No. | Action | Key point |
| --- | --- | --- |
| 1 | Check that the burners went off on the air interlock and the gas safety valves closed. | ⚠ Safety |
| 2 | Stop pushing; keep the doors closed; record the pause of the push cycle. | ● Critical |
| 3 | Inform maintenance: blower motor, belt, bearing, filter, damper (see the predictive maintenance checksheet F-MT-01 history). |  |
| 4 | Tag the trays in the zones that left their tolerance as suspect (SOP-HT-08); record the alarm. | ◆ Quality |

Product in the line

| Where the tray was | Risk and disposition |
| --- | --- |
| Zones that left their tolerance | As EI-HT-05: hold and test before release. |

Restart only when

- Blower repaired and air pressure normal; purge and relight by the burner management sequence; zones within tolerance and soaked.

# EI-HT-09  Controlled cooling chamber failure (isothermal annealing)

| Recognised by | Cooling blower stopped, cooling timer / damper fault, a tray not transferred in or out of the cooling chamber, the cooling cycle not finished before the next push. |
| --- | --- |
| Alarm on the CALOR Alarms page | Controlled cooling air / blower failure (isothermal annealing) |
| Furnaces | F-01, F-02, F-03, F-04 |

Immediate actions

| No. | Action | Key point |
| --- | --- | --- |
| 1 | Stop pushing: the next tray must not leave the hardening chamber while the cooling chamber is not working. | ● Critical |
| 2 | Note the tray in the cooling chamber and the minutes of cooling it got. | ◆ Quality |
| 3 | Inform maintenance (blower, damper, rail transfer, timer). |  |
| 4 | Tag the tray in the cooling chamber as suspect; record the alarm and the pause of the push cycle. | ◆ Quality |

Product in the line

| Where the tray was | Risk and disposition |
| --- | --- |
| Tray in the controlled cooling chamber | Wrong cooling: hold; hardness and microstructure (ferrite–pearlite, no bainite / martensite) before release. |
| Trays in the hardening chamber | Longer at temperature during the stop: within the specification usually; check decarburization after a long stop. |

Restart only when

- Blower, damper, timer and transfer working; the cooling cycle (transfers + cooling) fits in the push time of the job card.

# EI-HT-10  Pusher, extractor or rail transfer jam

| Recognised by | Pusher stalls or its hydraulic pressure rises sharply, extractor or transfer stops, a tray tilted / stuck, a push not completed. |
| --- | --- |
| Alarm on the CALOR Alarms page | Pusher / tray transport failure (jam, tray stuck) |
| Furnaces | F-01, F-02, F-03, F-04 |

Immediate actions

| No. | Action | Key point |
| --- | --- | --- |
| 1 | Stop the drive (emergency stop); never push harder or by hand into a hot chamber. | ⚠ Safety |
| 2 | Inform the shift supervisor and maintenance; note the time and the tray positions. | ◆ Quality |
| 3 | Keep the doors closed while the cause is found; recover a stuck tray only by the furnace manual's method with the burners in the state it requires. | ⚠ Safety |
| 4 | Record the pause of the push cycle; record the alarm. | ◆ Quality |
| 5 | After clearing: check trays and hearth rails for damage; inspect the trays involved (Load build page). |  |

Product in the line

| Where the tray was | Risk and disposition |
| --- | --- |
| All trays in the line | Stayed longer than the job card at temperature. Hold the trays whose time exceeded the job card tolerance; hardness (and decarburization for long stops) before release. Tilted tray: parts may have fallen – trap point check of the chamber at the next shutdown (Trap points page). |

Restart only when

- Cause removed; trial push with an empty tray; pusher pressure back to normal (F-MT-01).

# EI-HT-11  Control, SCADA or thermocouple failure

| Recognised by | Thermocouple break or implausible reading, the two wall readings of a zone far apart, controller fault, SCADA screen frozen or recording stopped. |
| --- | --- |
| Alarm on the CALOR Alarms page | Control thermocouple break / reading out of range · SCADA / recorder failure |
| Furnaces | F-01, F-02, F-03, F-04 |

Immediate actions

| No. | Action | Key point |
| --- | --- | --- |
| 1 | Compare the two wall thermocouples of the zone and the over-temperature controller reading; run the zone on the healthy sensor or stop pushing as the furnace instruction allows. | ● Critical |
| 2 | SCADA recording stopped: write the zone readings by hand in the shift log every 15 minutes until it runs again (the CQI-9 record must not have a gap). | ◆ Quality |
| 3 | Replace a failed thermocouple; do a SAT after the replacement (Pyrometry → SAT, replacement record). | ◆ Quality |
| 4 | Tag the trays in the zone from the last plausible reading until the zone was controlled again; record the alarm. | ◆ Quality |

Product in the line

| Where the tray was | Risk and disposition |
| --- | --- |
| Trays in the zone without a valid reading | Hold; hardness before release. |

Restart only when

- Valid control and recording on both walls of every zone; SAT done after a thermocouple replacement.

# EI-HT-12  Over-temperature trip

| Recognised by | Over-temperature controller tripped, burners of the chamber off. |
| --- | --- |
| Alarm on the CALOR Alarms page | Over-temperature protection tripped |
| Furnaces | F-01, F-02, F-03, F-04 |

Immediate actions

| No. | Action | Key point |
| --- | --- | --- |
| 1 | Do not reset the trip before the cause is known; inform the shift supervisor and maintenance. | ⚠ Safety |
| 2 | Find the cause: control thermocouple, controller, set value, burner valve stuck open. |  |
| 3 | Reset only with the shift supervisor; restart by WI-HT-02. | ⚠ Safety |
| 4 | Tag every tray in the chamber at the trip; record the alarm. | ◆ Quality |

Product in the line

| Where the tray was | Risk and disposition |
| --- | --- |
| Every tray in the tripped chamber | Overheated possibly: grain growth, decarburization, overheating. Hold; hardness, microstructure and grain size before release. |

Restart only when

- Cause corrected; over-temperature protection tested; zones within tolerance and soaked.

# 8. Equipment that must be available

- Main gas isolation valve outside the bay, marked and reachable; its location written in EI-HT-05.

- CO₂ / foam extinguishers at the F-02 oil quench and the hydraulic units; quench hood / lid working; no water extinguisher at the oil quench.

- Manual / emergency drive of the quench elevator (F-02) and its instruction at the quench.

- UPS for the SCADA and the burner management system; standby generator for the critical loads if the site has one (to confirm).

- Red HOLD tags, a marked quarantine area, a torch at each control panel, the shift log for manual readings.

# 9. Training and drills

Every operator, supervisor and maintenance technician of the line is trained on this procedure before working alone and after each revision (training record on the Procedures & WIs page). Drills keep the first actions automatic:

| Drill | How often | Record |
| --- | --- | --- |
| Power failure (walk-through at the line, every shift team) | Every 6 months | Drill record: date, team, time to safe state, gaps found |
| Gas leak evacuation | Every 12 months (with the site fire drill) | Site emergency drill record |
| Quench oil fire (hood, extinguisher) | Every 12 months | Drill record; extinguisher inspection tag |
| Standby supply start (if installed) | Monthly test run | Generator log |

# 10. Records

- Alarm / emergency record with the reaction plan – CALOR, Process → Alarms.

- Pause of the push cycle and the trays in the line – CALOR, Production → Daily production.

- Suspect product and its disposition – the reaction plan of the record, the Hardness page, the Rework page.

- Restart – Start-up checksheet F-HT-02A; equipment repair – maintenance work order; thermocouple replacement and SAT – Pyrometry → SAT.

- Drill records and training records.

# 11. Related documents

WI-HT-01 General operating procedure · WI-HT-02 Furnace start-up · WI-HT-03 Furnace shut-down · SOP-HT-08 Segregation of suspect and nonconforming product · SOP-HT-09 Product inspection and testing · SOP-QA-12 Rework and reprocessing · Alarm response matrix (CALOR Alarms page) · Quench media test plan · F-HT-02A / F-HT-03A Start-up / shutdown checksheets · F-MT-01 Predictive maintenance checksheet · Site emergency plan · Furnace and burner manufacturers' manuals.

# 12. Revisions

| Rev | Approved | Approved by | Reason for the change |
| --- | --- | --- | --- |
| 0 |  |  | First issue (includes EI-HT-04 to EI-HT-12) |

| Prepared by (HT Engineer) Sign · Date | Approved by (HT Manager) Sign · Date | Reviewed (Maintenance Head / Safety) Sign · Date |
| --- | --- | --- |