DRAFT for review: confirm the steps, heat-up rates and the burner sequence against the furnace and burner manufacturers' manuals and the gas safety rules, then approve.

| MASS WIRE AND STEELS PVT. LTD. · BHIWADI Work instruction · Furnace start-up | DOCUMENT WI-HT-02 | REVISION 0 | APPROVED __ / __ / ____ |
| --- | --- | --- | --- |

| Covers | Equipment start-up (CQI-9 HTSA 1.15) |
| --- | --- |
| Applies to | Furnaces F-01, F-02, F-03 and F-04: hardening chamber, controlled cooling chamber, tempering chamber; quench tanks F-01 (water) and F-02 (oil) |
| Used by | Furnace operator, Shift supervisor, Maintenance technician, HT Manager |
| Record | Start-up checksheet F-HT-02A; start-up record on the CALOR Alarms page |
| Kept at | Control panel of each furnace; HT office; Procedures & WIs page in CALOR |
| Language | English, Hindi |

# 1. Purpose

Start a furnace line safely and bring it to the process conditions of the job card before the first tray is pushed, so that no tray is heat treated in a furnace that is not ready.

# 2. Scope

Every start-up of a pusher furnace line: cold start, restart after a power failure, after maintenance, after an alarm stop and from hot standby. Shut-down is WI-HT-03; emergencies are SOP-HT-11. This instruction does not replace the furnace and burner manufacturers' manuals.

# 3. Responsibilities

- Shift supervisor: decides the start-up, checks the pre-conditions, signs the checksheet.

- Furnace operator: carries out the steps and fills in the checksheet.

- Maintenance technician: gas, burner and electrical checks; confirms the equipment is fit after maintenance.

- HT Manager: releases the furnace for production after a restart from an emergency or a maintenance change; decides on product left inside.

# 4. Safety

- Only trained and authorised persons light a furnace. Never bypass a purge, an interlock, a flame safeguard or a gas pressure switch.

- Gas smell at any time: stop, do not switch anything, follow SOP-HT-11 (EI-HT-05 part B).

- Heat-resistant gloves, face shield, apron and safety shoes at the doors and the quench; keep clear of moving equipment while it is tested.

# 5. Types of start-up

| Type of start-up | What is different |
| --- | --- |
| Cold start (from ambient) | All steps. Heat-up from cold at the furnace manual's rate with holds; the longest start-up. Check that no tray was left inside. |
| Restart after power failure | Steps A–H. Read the emergency record (EI-HT-04) first; the trays left in the line are SUSPECT. Check the quench oil temperature and agitation before the first quench; check that every motor restarted in the right direction. |
| Restart after maintenance | Steps A–H. Record the change on the Equipment page; a TUS / SAT after a change to the burners, refractory, thermocouples, controllers or recorders before production if the Equipment page requires it. Run the changed equipment empty first. |
| Restart after an alarm stop (gas, air, over-temperature, quench, jam) | Steps A–H from the step after the cause was removed. Over-temperature trip: reset only with the shift supervisor (EI-HT-12). Follow the restart conditions of the emergency instruction. |
| Restart from hot standby / idle | The furnace was held hot without production: steps A (records), B, E, F, G, H. Bring the set values from the standby values to the job card; soak again before the first tray. |

# 6. Steps

Key points: ⚠ Safety · ● Critical (checksheet item: stop if not met) · ◆ Quality

| No. | Step | Key point |
| --- | --- | --- |
| A. Decide and prepare (shift supervisor) |  |  |
| 1 | Confirm the type of start-up (table in section 5) and the reason; for a restart after an emergency, read the emergency record on the CALOR Alarms page and the decision on the trays left inside. | ◆ Quality |
| 2 | Open the Start-up checksheet F-HT-02A for the furnace; every step marked ● below is an item of the checksheet. | ◆ Quality |
| 3 | Maintenance work closed: work orders signed off, permits returned, locks removed by their owners, guards and covers fitted, tools and materials out of the furnace and the pits. | ⚠ Safety |
| 4 | After maintenance or a rebuild of refractory, burners, thermocouples, controllers or recorders: record the change on CALOR → Pyrometry → Equipment; a TUS and / or SAT may be needed before production (the Equipment page says which). | ● Critical |
| 5 | Product left inside from the shutdown or the emergency: confirm its positions from the shutdown checksheet F-HT-03A; it stays tagged SUSPECT and goes to the HT Manager's decision (SOP-HT-08). | ◆ Quality |
| B. Checks before ignition |  |  |
| 6 | Gas supply: main isolation valve open by the authorised person; gas pressure at the furnace gas train within its range (record the value). | ● Critical |
| 7 | Combustion air blower started; air pressure normal; exhaust / flue fan running and draught normal. | ⚠ Safety |
| 8 | Burner safety: interlocks, flame detectors and gas pressure switches tested by the burner management test function (no bypass, no jumper). | ● Critical |
| 9 | Control thermocouples TC-01 … TC-06 of both chambers connected; both walls of every zone read plausibly at ambient / hot standby (odd = right wall, even = left wall). | ● Critical |
| 10 | SAT of the control thermocouples in date (CALOR → Pyrometry → SAT); TUS of the chamber in date for the set values to be used (CALOR → TUS). | ● Critical |
| 11 | Over-temperature protection set (above the highest set value + tolerance as the furnace instruction says) and active. | ● Critical |
| 12 | SCADA recording running and its clock correct; the controllers in automatic. | ● Critical |
| C. Purge and ignition |  |  |
| 13 | Start the purge from the burner management panel; wait for the full purge time — never shorten or bypass it. | ⚠ Safety |
| 14 | Ignite the burners only by the burner management sequence, zone by zone as the furnace manual says. Never light a burner by hand or with an open flame. | ⚠ Safety |
| 15 | A burner that does not light or locks out: stop, find the cause (EI-HT-05); do not repeat ignition attempts without a new purge. | ⚠ Safety |
| 16 | Note the time the burners were lit (checksheet). | ◆ Quality |
| D. Heat-up |  |  |
| 17 | Heat up at the rate of the furnace manual (refractory): from cold in steps with holds; after a refractory repair or a new lining follow the dry-out schedule of the refractory supplier. | ◆ Quality |
| 18 | Raise the set values in steps, not straight to the process set values, unless the manual allows it. | ◆ Quality |
| 19 | Watch the SCADA every 30 minutes during the heat-up: both walls of each zone rising together; a wall lagging behind points to a burner or a thermocouple fault. | ◆ Quality |
| 20 | During the heat-up check the pusher, extractor, doors, rail transfer and quench elevator by running them EMPTY. |  |
| E. Quench and controlled cooling |  |  |
| 21 | F-01 (water quench): water level within the marks, temperature within the limit, agitation / circulation running — agitation is not monitored, so check that the motor runs and the surface moves. | ● Critical |
| 22 | F-02 (oil quench): oil level within the marks, oil at 50–80 °C (heat it before the first quench if it is cold), agitation and oil cooler running, no water in the oil (crackle test if due — CALOR → Quench media). | ● Critical |
| 23 | F-03 / F-04: no quench; skip this part. |  |
| 24 | Every furnace running isothermal annealing: controlled cooling blower, dampers, rail transfer in / out and cooling timer tested with an empty tray. | ◆ Quality |
| F. Set values, stabilisation and soak |  |  |
| 25 | Take the job card (process specification in force, CALOR → Specifications) of the first part to be loaded; enter its set values for every zone of both chambers and its push time. Check that the specification is approved for this furnace. | ● Critical |
| 26 | Wait until every zone of both chambers is within the tolerance of its set value on BOTH wall thermocouples; note the time and the readings of all 12 thermocouples on the checksheet. | ● Critical |
| 27 | Soak: hold at the set values for at least the minimum soak (30 min, see the checksheet settings) after the time above before the first tray. | ● Critical |
| 28 | A zone that will not come into tolerance, or two walls far apart: do not load; inform maintenance (EI-HT-11 if a thermocouple is suspect). | ● Critical |
| G. First trays and release |  |  |
| 29 | Trays: use only trays in use with the inspection in date (CALOR → Load build); HK trays not cracked or distorted; load not above 200 kg. | ◆ Quality |
| 30 | Push the first production tray only after the soak; note the time on the checksheet and enter the tray on the Daily production page (start time and push time of the day). | ● Critical |
| 31 | Load the first batch to its loading standard; record the load check of its first tray. | ◆ Quality |
| 32 | First-off test: take the first tray's samples to the laboratory; hardness (and the tests of the specification) within 8 h of the first tray (checksheet setting). Hold the first batch until the result is OK. | ● Critical |
| 33 | First-off result not OK: stop pushing, hold the trays processed since the start-up (SOP-HT-08), inform the HT Manager. | ● Critical |
| H. Records and release |  |  |
| 34 | Complete the checksheet F-HT-02A: result RELEASED FOR PRODUCTION (or the reason it is not), signatures of the operator and the shift supervisor; the HT Manager signs the release after a restart from an emergency or a maintenance change. | ◆ Quality |
| 35 | Record the start-up on CALOR → Alarms (start-up record: type, reason, ready time, first push, checklist, first-off test). | ◆ Quality |
| 36 | Close the pause of the push cycle on the Daily production page with the restart time if the stop was recorded there. | ◆ Quality |

# 7. When something goes wrong

A critical step not met: do not light, or do not push; inform the shift supervisor and maintenance; record it on the checksheet and start a reaction plan on the CALOR Alarms page. Emergencies during the start-up: SOP-HT-11.

# 8. Records

- Start-up checksheet F-HT-02A (kept 3 years or as the record retention rule says).

- Start-up record on CALOR → Process → Alarms; the first tray and push time on the Daily production page; the first-off test on the Hardness page.

- Equipment change record and any TUS / SAT on the Pyrometry pages.

# 9. Related documents

WI-HT-01 General operating procedure · WI-HT-03 Furnace shut-down · SOP-HT-11 Emergency response · SOP-HT-08 Segregation of suspect and nonconforming product · SOP-HT-09 Product inspection and testing · Process specifications / job cards · F-HT-02A Start-up checksheet · F-MT-01 Predictive maintenance checksheet · Furnace and burner manufacturers' manuals.

# 10. Revisions

| Rev | Approved | Approved by | Reason for the change |
| --- | --- | --- | --- |
| 0 |  |  | First issue |

| Prepared by (HT Engineer) Sign · Date | Approved by (HT Manager) Sign · Date | Controlled copy issued to Location · Date |
| --- | --- | --- |